Simulation task

EXPERIMENT:

Multipath   channel: h = [1, 0, 0.6, 0, 0.3] (samples at 1 kHz). Input x = [1 0 0 0 0 1 0   0 0 0] (two pulses). In MATLAB compute y=conv(x,h). Plot x and y. Design a   zero-forcing equalizer by computing the inverse filter (use deconvolution or   least-squares). Apply equaliser and plot recovered signal. Compute bit error   rate proxy from residual ISI. give me matlab code

PROGRAM:

clc; clear; close all;

% Channel and input

h = [1 0 0.6 0 0.3];

x = [1 0 0 0 0 1 0 0 0 0];

% Multipath output

y = conv(x,h);

% Plot input and received signal

figure;

subplot(3,1,1);

stem(x,'filled'); grid on;

title('Input Signal x[n]');

subplot(3,1,2);

stem(y,'filled'); grid on;

title('Received Signal y[n]');

% Zero-Forcing equalizer using least squares

L = 7;

H = convmtx(h,L);

d = [1; zeros(size(H,1)-1,1)];

w = pinv(H)*d;

% Equalization

r = conv(y,w);

r = r(1:length(x));

% Plot recovered signal

subplot(3,1,3);

stem(r,'filled'); grid on;

title('Recovered Signal');

% Detection and BER proxy

xhat = r > 0.5;

BER = sum(xhat ~= x)/length(x);

fprintf('Equalizer coefficients:\n');

disp(w');

fprintf('BER proxy = %.4f\n',BER);

fprintf('Residual ISI = %.4f\n',sum((r-x).^2));

OUTPUT:

RESULT:

The zero-forcing equalizer effectively compensates for the multipath distortion and recovers both transmitted pulses correctly. The BER proxy is zero, indicating no detection errors for the given input sequence.

EXPERIMENT:

9	Define H(s) = 2*s/(s^2 + 2*s + 25) in Scilab. Compute frequency response using repfreq() over 0.1 to 100 rad/s. Plot magnitude (linear and dB) and phase. Extract the centre frequency (peak of |H(jw)|) and -3 dB bandwidth programmatically using find(). Annotate the plot.	BT4	10

PROGRAME:

clc; clear; close all;

% Transfer function

H = tf([2 0],[1 2 25]);

% Frequency range

w = logspace(-1,2,1000);

[mag,ph] = bode(H,w);

mag = squeeze(mag); ph = squeeze(ph);

fc = w(k);

idx = find(20*log10(mag) >= 20*log10(mp)-3);

BW = w(idx(end))-w(idx(1));

% Plot

figure;

subplot(2,1,1);

semilogx(w,mag); grid on;

hold on; xline(fc,'r--');

ylabel('|H(j\omega)|'); title('Magnitude Response');

subplot(2,1,2);

semilogx(w,20*log10(mag)); grid on;

hold on; yline(20*log10(mp)-3,'r--'); xline(fc,'r--');

= %.2f rad/s',fc,BW));

fprintf('Centre frequency = %.2f rad/s\n',fc);

fprintf('-3 dB Bandwidth = %.2f rad/s\n',BW);

fprintf('Peak magnitude = %.3f\n',mp);

OUTPUT:

RESULT:

The system exhibits a band-pass response with a centre frequency of approximately 5 rad/s and a -3 dB bandwidth of 2 rad/s. The magnitude and phase plots confirm the expected frequency-selective behavior.