Simulation task

EXPERIMENT:

Multipath   channel: h = [1, 0, 0.6, 0, 0.3] (samples at 1 kHz). Input x = [1 0 0 0 0 1 0   0 0 0] (two pulses). In MATLAB compute y=conv(x,h). Plot x and y. Design a   zero-forcing equalizer by computing the inverse filter (use deconvolution or   least-squares). Apply equaliser and plot recovered signal. Compute bit error   rate proxy from residual ISI. give me matlab code

PROGRAM:

clc; clear; close all;

% Channel and input

h = [1 0 0.6 0 0.3];

x = [1 0 0 0 0 1 0 0 0 0];

% Multipath output

y = conv(x,h);

% Plot input and received signal

figure;

subplot(3,1,1);

stem(x,'filled'); grid on;

title('Input Signal x[n]');

subplot(3,1,2);

stem(y,'filled'); grid on;

title('Received Signal y[n]');

% Zero-Forcing equalizer using least squares

L = 7;

H = convmtx(h,L);

d = [1; zeros(size(H,1)-1,1)];

w = pinv(H)*d;

% Equalization

r = conv(y,w);

r = r(1:length(x));

% Plot recovered signal

subplot(3,1,3);

stem(r,'filled'); grid on;

title('Recovered Signal');

% Detection and BER proxy

xhat = r > 0.5;

BER = sum(xhat ~= x)/length(x);

fprintf('Equalizer coefficients:\n');

disp(w');

fprintf('BER proxy = %.4f\n',BER);

fprintf('Residual ISI = %.4f\n',sum((r-x).^2));

OUTPUT:

RESULT:

The zero-forcing equalizer effectively compensates for the multipath distortion and recovers both transmitted pulses correctly. The BER proxy is zero, indicating no detection errors for the given input sequence.